Cranes Loads

Middle span Crane loads :

Crane capacity = 32t         , span = 38m       , wheel spacing = σ = 5.8 m
Max Wheel load = Pmax = 33 t           , Min wheel load = Pmin = 9.4t

VLL max = Pmax * ( 1 + ( S – σ ) / S ) = 35 t    , VLL min = 10 t     , VDL = 0.2 t/m\ * 6m / 2 = 0.6 t

BFmax = 1/7  * 2 * Pmax = 10t           , BFmin = 2.7 t

Hmax = 0.1 VLL = 0.4 t		, Hmin = 1t       …… Taken 1.9t

Vtmax = 1.25 VLL + VDL = 45 t …. Taken 40t	, Vtmin = 14t    …. Taken 19t

Left span crane loads :

Capacity = 10t  >>>>> 16 t    , crane span = 24 m          , wheel spacing = σ = 4.56 m

CTG span = S = 6m
Max wheel load = Pmax =  8 t

Minimum Wheel load = Pmin = 2.4t

Max Lateral shock = 0.1 * 8t = 0.8t       ….. taken 1.15 t

Min Lateral shock = 0.31 t   …… taken 0.7t

VDL = 0.2t/m\ * 6m /2 = 0.6t

VLL = Pmax + Pmax * ( S - σ ) / S = 10 t

Vtmax = 1.25 VLL + VDL = 13t  …… taken 11.5t 		, Vtmin = 3.6t  ….. taken 7.2t

BFmax = 2/7  * Pmax = 2.3t     …. Taken 3.3t

Corrugated Sheets

roof Corrugated sheets :

Middle span Live load = 60 – 200/3 * tan(α) = 60 – 200/3  * 0.06 = 56 kg / m2

- use corrugated sheet of allowable live load = 100 kg / m2 , span 2 m
use continuous corrugated sheet of thickness 0.55 mm

- Ow Dead Load = 5.25 kg/m2

- Total load  Gravity = 5.25 + 56 = 61.25 kg/m2

- Wind Load ( wind side ) = q * c * K * a / cos(angle) = 50kg/m2 * -0.8 * 1.15 / cos(tan-1(1200/20000) ) = -46 kg/m2

- Wind Load ( wind opposite side ) = q * c * K * a / cos(angle) = 50kg/m2 * -0.5 * 1.15 / cos(tan-1(1200/20000) ) = -29 kg/m2

side spans live load = 60 – 200/3 * tan(α) = 60 – 200/3  * 0.1 = 53 kg / m2

- use corrugated sheet of allowable live load = 100 kg / m2  , span 2.5 m

- use continuous corrugated sheet of thickness 0.7 mm

- ow = 6.66 kg/m2

- Total load = 6.66 + 53 = 60 kg/m2

- Wind Load = q * c * K * a / cos(angle) = 50kg/m2 * -0.8 * 1.15 / cos(tan-1(1200/12500) ) = -46 kg/m2

- Wind Load ( wind side ) = q * c * K * a / cos(angle) = 50kg/m2 * -0.5 * 1.15 / cos(tan-1(1200/20000) ) = -29 kg/m2

side Corrugated sheets :

level > 10 m , Span = 2 m

- wind load ( wind direction ) = Ce * K * q = 0.8 * 1.15 * 50 = 50 kg / m2  ,

- Wind load ( opposite wind direction )  = = Ce * K * q = -0.5 * 1.15 * 50 = -32 kg / m2  ,

- Ow = 4.75 kg/m2 (vertical load)

- Use continuous corrugated sheets for all side of thickness 0.5 mm

level < 10 m , Span = 2.5 m

- Use corregated sheet of allowable load = 50 kg / m2

- wind load ( wind direction ) = Ce * K * q = 0.8 * 1.0 * 50 = 40kg/m2

- wind load ( wind opposite dir. ) = Ce * K * q = -0.5 * 1.0 * 50 = -25kg/m2

- Ow = 4.75 kg/m2 (vertical load)

- Use continuous corrugated sheets for all side of thickness 0.5 mm

Corrugated Sheets Summery :

- Use continuous in middle span roof corrugated sheets of 0.7 mm

- Use continuous in side spans roof corrugated sheets of 0.55 mm

- Use continuous in side corrugated sheets of 0.50 mm

Mezanin

Flooring = 200 kg/m2

Storage Floor Live Load = 500 Kg/m2

Management Floors Live Load = 400 Kg/m2

Walls distributed load = 200 kg/m2

Deck span = 2.50 m

Use Metal Deck thickness = 1.2 mm

For Storage Floor Use concrete thickness = 8 cm

- concrete load = 2500 kg/m3 * 0.08 = 200 kg/m2

- total dead load = 600 kg/m2

- total live load = 500 kg/m2

- total working load = 1100 kg/m2

- Total ultimate load for storage floor = 1.4 * (200 + 200 + 200 ) + 1.6 * 500 = 1640 kg/m2

- Allowable load for storage floor = 1758 kg/m2

For management Floor Use concrete thickness = 7 cm

- concrete load = 2500 kg/m3 * 0.08 = 200 kg/m2

- total dead load = 575 kg / m2

- total live load = 400 kg/m2

- total working load = 975 kg/m2

- Total ultimate load for mang. floor = 1.4 * (200 + 175 + 200) + 1.6 * 400 = 1445  kg/m2

- Allowable load for management floor = 1542 kg/m2

Mezanin Summery :

Use Metal Deck of 1.2 mm

For Management Floors , Concrete Thickness = 7 cm

For storage Floor , Concrete Thickness = 8 cm

Roof Purlins Design

Purlins Middle span :

- Live load = 56 kg/m2 * 2m = 112 kg/m

- Ow = 25 kg/m

- Dead load = 5.25 kg/m2 * 2m + 25kg/m = 30.5 kg/m

- Dead Load Reactions = 30.5kg/m * 6m / 2 = 91.5 kg = 0.092t

- Live load Reactions = 112kg/m * 6m / 2 = 336 kg = 0.34t

- Total load Reactions = 427.5 kg = 0.43 t

- Total load = 30.5kg/m + 112kg/m = 0.15 t/m\  …….. critical purlins design load

- Wind Load ( wind direction ) = -46kg/m2 * 2m = -92 kg/m\  = -0.1 t/m\

- Wind Load ( opposite direction ) = -29kg/m2 * 2 = -0.06 t/m\

- Wind Load ( wind direction ) Reaction = -46kg/m2 * 2m * 6m /2 = -0.28t

- Wind load (opposite wind dir) Reaction = -29kg/m2 * 2m * 6m /2 = -0.18t

- Frame span = 6m  ->  Max Moment = 0.15 * 62 / 8 = 0.68 t.m = 68 t.cm

Purlins Side Spans :

- Live Load = 53 kg/m2 * 2.5m = 132.5 kg/m

- Ow = 25 kg/m

- Dead Load = 25kg/m + 6.66kg/m2 * 2.5m = 41.65  kg/m

- Dead load Reactions = 41.65kg/m * 6m / 2 = 125 kg = 0.125 t @ 2.5m span

- Live load Reactions = 132.5kg/m * 6m / 2 = 397.5 kg = 0.4t @ 2.5m span

- Total load Reaction = 0.525t

- Total Load = 41.65 kg/m + 132.5 kg/m = 0.18 t/m\ …... Critical purlins design load

- Wind Load ( wind direction ) = -46kg/m2 * 2m = -92 kg/m\  = -0.1 t/m\

- Wind Load ( opposite direction ) = -29kg/m2 * 2 = -0.06 t/m\

- Wind Load ( wind direction ) Reaction = -46kg/m2 * 2.5m * 6m /2 = -0.35t

- Wind load (opposite wind dir) Reaction = -29kg/m2 * 2.5m * 6m /2 = -0.22t

- Frame span = 6m  ->  Max Moment = 0.18 * 62 / 8 = 0.81 t.m = 81 t.cm

Side Purlins

Side Purlins Beams  < 10.00 m level:

- Wind Lateral Load (wind direction) = 40kg/m2 * 2.5m = 100kg/m = 0.1 t/m …..critical purlins design load

- Wind Lateral Load (opposite wind dir) = -25kg/m2 * 2.5m = 63kg/m = 0.063 t/m

- Vertical load = corrugated sheet ow + purlin ow = 4.75 kg/m2 * 2.5m + 25kg/m = 36.9kg/m = 0.04 t/m\

- Lateral Reaction (wind direction) = 0.1t/m * 6m / 2 = 0.3t

- Later reaction (opposite wind dir )  = 0.063t/m * 6m /2 = 0.19 t

- Vertical Reaction = 36.9kg/m * 6m / 2 = 0.11t

- Max moment = 0.45 t.m = 45 t.cm ,

Side Purlins Beams  > 10.00 m level: ( design like purlins at level below 10.00m)

- Wind Lateral Load (wind direction) = 46kg/m2 * 2 m = 92kg/m = 0.092 t/m

- Wind Lateral Load (opposite wind dir) = -32kg/m2 * 2 m = 64kg/m = 0.064 t/m

- Vertical load = corrugated sheet ow + purlin ow = 4.75 kg/m2 * 2 m + 25kg/m = 35kg/m = 0.035 t/m\

- Lateral Reaction (wind dir) = 0.092t/m * 6m / 2 = 0.28t

- Lateral Reaction (opposite wind dir) = 0.064t/m * 6m / 2 = 0.19t

- Vertical Reaction = 35kg/m * 6m / 2 = 0.11t

Beams load

Mezanin storage floor main beams (using metal deck allowable load ) :

- Dead load = 1.6t/m * 6m = 9.6 t , secondary beams from one side , @2.5m spacing

- Live load = 1.25t/m * 6m = 7.5 t , secondary beams from one side , @2.5m spacing

Mezanin management floor main beams (using metal deck allowable load ) :

- Dead load = 1.55t/m * 6m = 9.3 t, secondary beams from one side , @2.5m spacing

- Live load = 1t/m * 6m = 6 t , secondary beams from one side , @2.5m spacing

Mezanin Storage Floor Secondary Beams :

- Dead load = 0.6t/m2 * 2.5m = 1.5 t/m

- Total Live load = 0.5t/m2 * 2.5m = 1.25 t/m

- ow = 100 kg / m

- Total Dead load = 0.1t/m + 1.5t/m = 1.6 t/m

Mezanin Management Floor Secondary Beams :

- Dead load = 0.58t/m2 * 2.5m = 1.45 t/m

- Ow = 0.1 t/m

- Total Dead load = 1.55 t/m

- Live load = 0.4t/m2 *  2.5m = 1 t/m

Frame girder load :

- Purlins concentrated Dead loads from each sides= 2 * 0.125t = 0.25t  ,9purlins@2.5m

- Purlins concentrated Live loads from each sides = 2 * 0.4t = 0.8t ,9purlins@2.5m

- Utilities Dead loads = 0.1 t /m

Truss loads

- Purlins concentrated Dead loads at nodes from each sides = 2* 0.092t = 0.185t ,purlins@2.5m at each node

- Purlins concentrated Live loads at nodes from each sides = 2* 0.34t = 0.68t ,purlins@2.5m at each node

- Utilities Dead Load = 100 kg / node = 0.1t /node

End Gable Purlins

Wind load = 46kg/m2  -> span = 2m

Wind load = 40 kg/m2  -> span = 2.5m

Use corrugated sheets of thickness = 0.5 mm , weight = 4.8 kg/m2

Used purlins :

Stairs

Live Load = 500 kg/m2

Dead load on Carriage = 100 kg/m\

Steps Calculations :

Stair width = 2.5 m

stair Tread = 267 mm

stair riser = 167 mm

distributed load on Tread = 0.267m * 0.5t/m2 = 0.134 t/m\

Moment on Tread = 0.134t/m\ * 2.52 / 8 = 0.105 t m

Shear on Tread = 0.17 tons

Take Tread Plate Thickness = 1.2 cm

Using two Bolts to fixing Tread with Angle of Diameters = 10 mm

Stress due to moment = Mx * tp / ( 2 Ix ) = 6 Mx / ( tp2 * width ) = 0.105 * 100 / ( 1.22 * (26.7 - 2)) =  1.77 t/cm2  < ( 0.58 * fy = 2.1 t/cm2 )       ……… ok

Stress due to shear = Q / ( tp * width ) = 0.17 / ( 1.2 * ( 26.7 - 2 ) ) = almost zero

Step Fixation Angle Calculations :

Bolts construction conditions = 3 * bolts diameter * number of bolts = 60 mm

Welding length = Q / ( allowable weld stress * Sw ) = 0.17 / (0.2 * 5.2 * 0.4 ) = 0.35 cm = 3.5 mm

Use angle 50*5*5 of length 10 cm

Landing Fixation Angle Calculations :

Shear = 0.5t/m2 * 1.8m * 2.5m / 2 = 1.125 tons

Welding length = Q / ( allowable weld stress * Sw ) = 1.125 / (0.2 * 5.2 * 0.4 ) = 2.7 cm

Minimum angle length = 2.7 cm