Cranes Loads

Middle span Crane loads :

Crane capacity = 32t         , span = 38m       , wheel spacing = σ = 5.8 m
Max Wheel load = Pmax = 33 t           , Min wheel load = Pmin = 9.4t

VLL max = Pmax * ( 1 + ( S – σ ) / S ) = 35 t    , VLL min = 10 t     , VDL = 0.2 t/m\ * 6m / 2 = 0.6 t

BFmax = 1/7  * 2 * Pmax = 10t           , BFmin = 2.7 t

Hmax = 0.1 VLL = 0.4 t		, Hmin = 1t       …… Taken 1.9t

Vtmax = 1.25 VLL + VDL = 45 t …. Taken 40t	, Vtmin = 14t    …. Taken 19t

Crane Grider :

Assume that girder resist only one wheel as wheel spacing is 5.8 m

moment due to load = 1.25 x ( Pmax x girder length / 4 ) = 61.5 t.m -> 1.25 impact factor

moment due to ow = 0.1t/m x 62 / 8 = 0.45 t.m

Mx = total moment = 62 t.m

assume web dimensions = 300 x 10 mm  , flange thickness = 20 mm

web depth / web thickness = 30 < 190 / fy0.5    , safe for local buckling

required inertia = 53142 cm4

flange length = 400 mm   ,  flange length /  2x thickness = 10  < 21 / fy0.5…. Safe for local buckling

allowable shear = web area * yield stress = 30 x 1 x 2.1 = 60 tons > Pmax  …….safe

My = Hmax x girder length / 4 = 0.4 x 6 / 4 = 0.6 t.m     -> resisted by upper plate only

Plate inertia = 403 x 2 /12 = 10666 cm4

My allowable = stress x Ix /y = 2.1 x 10666 / 20 = 1120 t.cm = 11.2 t.m     …… safe

Deflection calculations :

Deflection = P L3 / 3EI = 0.22 cm

Allowable deflection = span / 800 = 0.75 cm …….. safe

Crane bracing :

Design on Breaking force

Design force = 10 / cos(45) = 14.1 tons

Lu = 1.1 m , use bracing of HSS 100X4 ,

Area = 15.5 cm2 , inertia = 236cm4 , ix = 3.9 cm , Lx/ix = 28.6

Allowable stress = 1.2 t/cm2

Applied stress = 0.91 t/cm2 ………… safe

Use weld of 4 mm

Weld length for box and plate = 10 / ( 0.2 x 5.2 x 0.4 ) + 2 x 0.4 = 24cm

Each weld line length for box section = 24cm / 4 = 6 cm

Gusset plate weld length = 20 cm from both sides , with thickness 5 mm

Gusset plate weld -> shear stress = normal stress = 10 / ( 0.5 x 2 x 20 ) = 0.5 t/cm2

Gusset plate weld combined stress = 1 ton/cm2    < 1.1 x 1.04 t/cm2 …… safe

Left span crane loads :

Capacity = 10t  >>>>> 16 t    , crane span = 24 m          , wheel spacing = σ = 4.56 m

CTG span = S = 6m
Max wheel load = Pmax =  8 t

Minimum Wheel load = Pmin = 2.4t

Max Lateral shock = 0.1 * 8t = 0.8t       ….. taken 1.15 t

Min Lateral shock = 0.31 t   …… taken 0.7t

VDL = 0.2t/m\ * 6m /2 = 0.6t

VLL = Pmax + Pmax * ( S - σ ) / S = 10 t

Vtmax = 1.25 VLL + VDL = 13t  …… taken 11.5t 		, Vtmin = 3.6t  ….. taken 7.2t

BFmax = 2/7  * Pmax = 2.3t     …. Taken 3.3t

Crane Grider :

Assume that beam resists only one crane wheel as wheels spacing is 4.56 m

Max moment = 0.1t/m * 62 / 8 + 8 * 6 /4 = 12.5 tons

web dimensions = 200 x 10 , web depth / thickness < 190/fy0.5          …… safe for local buckling

use flange thickness = 20 mm

required inertia = 7143 cm4

flange length = 12 cm       , flange length / thickness < 21/fy0.5        … safe

deflection = P L3 / 3EI = 0.4 cm

allowable deflection = span / 800 = 0.75 cm  ….. . safe

required bracket :

moment = Pmax x (1 + 4.56 / 6 ) x 0.3 x 1.25 = 5.3 t.m

use web dimension = 150 x 10 mm , flange thickness = 10

required inertia = 2145 cm4

flange length = 15 cm   -> flange length / 2 x thickness  = 12.5 < 21 /fy0.5  ….. safe for L.B

Crane Bracing Calculations :

Lateral shock = ( 1.15 + 3.3 ) / cos(45) = 6.3 t

Lu = 1 m , use bracing of HSS 100X4 ,

Area = 15.5 cm2 , inertia = 236cm4 , ix = 3.9 cm , Lx/ix = 25.6

Allowable stress = 1.3 t/cm2

Applied stress = 0.41 t/cm2 ………… safe

Use weld of 4 mm

Weld length for box and plate = 6.3 / ( 0.2 x 5.2 x 0.4 ) + 2 x 0.4 = 16cm

Weld length for each weld line in box section = 16 / 4 = 4 cm , take it 5 cm

Weld for gusset plate , length = 100 mm , thickness 4 mm , from both sides

Shear stress on gusset plate weld = normal stress = (1.15 + 3.3) / (10 x 0.4 x 2) = 0.56 t/cm2

Combined stress for gusset plate weld = 1.125 t/cm2  < 1.1 x 1.04 t/cm2 …..safe

Corrugated Sheets

roof Corrugated sheets :

Middle span Live load = 60 – 200/3 * tan(α) = 60 – 200/3  * 0.06 = 56 kg / m2

- use corrugated sheet of allowable live load = 100 kg / m2 , span 2 m
use continuous corrugated sheet of thickness 0.55 mm

- Ow Dead Load = 5.25 kg/m2

- Total load  Gravity = 5.25 + 56 = 61.25 kg/m2

- Wind Load ( wind side ) = q * c * K * a / cos(angle) = 50kg/m2 * -0.8 * 1.15 / cos(tan-1(1200/20000) ) = -46 kg/m2

- Wind Load ( wind opposite side ) = q * c * K * a / cos(angle) = 50kg/m2 * -0.5 * 1.15 / cos(tan-1(1200/20000) ) = -29 kg/m2

side spans live load = 60 – 200/3 * tan(α) = 60 – 200/3  * 0.1 = 53 kg / m2

- use corrugated sheet of allowable live load = 100 kg / m2  , span 2.5 m

- use continuous corrugated sheet of thickness 0.7 mm

- ow = 6.66 kg/m2

- Total load = 6.66 + 53 = 60 kg/m2

- Wind Load = q * c * K * a / cos(angle) = 50kg/m2 * -0.8 * 1.15 / cos(tan-1(1200/12500) ) = -46 kg/m2

- Wind Load ( wind side ) = q * c * K * a / cos(angle) = 50kg/m2 * -0.5 * 1.15 / cos(tan-1(1200/20000) ) = -29 kg/m2

side Corrugated sheets :

level > 10 m , Span = 2 m

- wind load ( wind direction ) = Ce * K * q = 0.8 * 1.15 * 50 = 50 kg / m2  ,

- Wind load ( opposite wind direction )  = = Ce * K * q = -0.5 * 1.15 * 50 = -32 kg / m2  ,

- Ow = 4.75 kg/m2 (vertical load)

- Use continuous corrugated sheets for all side of thickness 0.5 mm

level < 10 m , Span = 2.5 m

- Use corregated sheet of allowable load = 50 kg / m2

- wind load ( wind direction ) = Ce * K * q = 0.8 * 1.0 * 50 = 40kg/m2

- wind load ( wind opposite dir. ) = Ce * K * q = -0.5 * 1.0 * 50 = -25kg/m2

- Ow = 4.75 kg/m2 (vertical load)

- Use continuous corrugated sheets for all side of thickness 0.5 mm

Corrugated Sheets Summery :

- Use continuous in middle span roof corrugated sheets of 0.7 mm

- Use continuous in side spans roof corrugated sheets of 0.55 mm

- Use continuous in side corrugated sheets of 0.50 mm

Mezanin

Flooring = 200 kg/m2

Storage Floor Live Load = 500 Kg/m2

Management Floors Live Load = 400 Kg/m2

Walls distributed load = 200 kg/m2

Deck span = 2.50 m

Use Metal Deck thickness = 1.2 mm

For Storage Floor Use concrete thickness = 8 cm

- concrete load = 2500 kg/m3 * 0.08 = 200 kg/m2

- total dead load = 600 kg/m2

- total live load = 500 kg/m2

- total working load = 1100 kg/m2

- Total ultimate load for storage floor = 1.4 * (200 + 200 + 200 ) + 1.6 * 500 = 1640 kg/m2

- Allowable load for storage floor = 1758 kg/m2

For management Floor Use concrete thickness = 7 cm

- concrete load = 2500 kg/m3 * 0.08 = 200 kg/m2

- total dead load = 575 kg / m2

- total live load = 400 kg/m2

- total working load = 975 kg/m2

- Total ultimate load for mang. floor = 1.4 * (200 + 175 + 200) + 1.6 * 400 = 1445  kg/m2

- Allowable load for management floor = 1542 kg/m2

Mezanin Summery :

Use Metal Deck of 1.2 mm

For Management Floors , Concrete Thickness = 7 cm

For storage Floor , Concrete Thickness = 8 cm

Roof Purlins Design

Purlins Middle span :

- Live load = 56 kg/m2 * 2m = 112 kg/m

- Ow = 25 kg/m

- Dead load = 5.25 kg/m2 * 2m + 25kg/m = 30.5 kg/m

- Dead Load Reactions = 30.5kg/m * 6m / 2 = 91.5 kg = 0.092t

- Live load Reactions = 112kg/m * 6m / 2 = 336 kg = 0.34t

- Total load Reactions = 427.5 kg = 0.43 t

- Total load = 30.5kg/m + 112kg/m = 0.15 t/m\  …….. critical purlins design load

- Wind Load ( wind direction ) = -46kg/m2 * 2m = -92 kg/m\  = -0.1 t/m\

- Wind Load ( opposite direction ) = -29kg/m2 * 2 = -0.06 t/m\

- Wind Load ( wind direction ) Reaction = -46kg/m2 * 2m * 6m /2 = -0.28t

- Wind load (opposite wind dir) Reaction = -29kg/m2 * 2m * 6m /2 = -0.18t

- Frame span = 6m  ->  Max Moment = 0.15 * 62 / 8 = 0.68 t.m = 68 t.cm

Purlins Side Spans :

- Live Load = 53 kg/m2 * 2.5m = 132.5 kg/m

- Ow = 25 kg/m

- Dead Load = 25kg/m + 6.66kg/m2 * 2.5m = 41.65  kg/m

- Dead load Reactions = 41.65kg/m * 6m / 2 = 125 kg = 0.125 t @ 2.5m span

- Live load Reactions = 132.5kg/m * 6m / 2 = 397.5 kg = 0.4t @ 2.5m span

- Total load Reaction = 0.525t

- Total Load = 41.65 kg/m + 132.5 kg/m = 0.18 t/m\ …... Critical purlins design load

- Wind Load ( wind direction ) = -46kg/m2 * 2m = -92 kg/m\  = -0.1 t/m\

- Wind Load ( opposite direction ) = -29kg/m2 * 2 = -0.06 t/m\

- Wind Load ( wind direction ) Reaction = -46kg/m2 * 2.5m * 6m /2 = -0.35t

- Wind load (opposite wind dir) Reaction = -29kg/m2 * 2.5m * 6m /2 = -0.22t

- Frame span = 6m  ->  Max Moment = 0.18 * 62 / 8 = 0.81 t.m = 81 t.cm ,

- Zx = 57cm3

Side Purlins

Side Purlins Beams  < 10.00 m level:

- Wind Lateral Load (wind direction) = 40kg/m2 * 2.5m = 100kg/m = 0.1 t/m …..critical purlins design load

- Wind Lateral Load (opposite wind dir) = -25kg/m2 * 2.5m = 63kg/m = 0.063 t/m

- Vertical load = corrugated sheet ow + purlin ow = 4.75 kg/m2 * 2.5m + 25kg/m = 36.9kg/m = 0.04 t/m\

- Lateral Reaction (wind direction) = 0.1t/m * 6m / 2 = 0.3t

- Later reaction (opposite wind dir )  = 0.063t/m * 6m /2 = 0.19 t

- Vertical Reaction = 36.9kg/m * 6m / 2 = 0.11t

- Max moment = 0.45 t.m = 45 t.cm ,

Side Purlins Beams  > 10.00 m level: ( design like purlins at level below 10.00m)

- Wind Lateral Load (wind direction) = 46kg/m2 * 2 m = 92kg/m = 0.092 t/m

- Wind Lateral Load (opposite wind dir) = -32kg/m2 * 2 m = 64kg/m = 0.064 t/m

- Vertical load = corrugated sheet ow + purlin ow = 4.75 kg/m2 * 2 m + 25kg/m = 35kg/m = 0.035 t/m\

- Lateral Reaction (wind dir) = 0.092t/m * 6m / 2 = 0.28t

- Lateral Reaction (opposite wind dir) = 0.064t/m * 6m / 2 = 0.19t

- Vertical Reaction = 35kg/m * 6m / 2 = 0.11t

- Moment = 0.092 t/m * 62 / 8 = 0.414 t.m = 41 t.cm

- Zx = 41 t.cm / 0.58*fy = 41/ 1.4 = 29.3 cm3

Beams load

Mezanin storage floor main beams (in  plan) (using metal deck allowable load ) :

- Dead load from secondary beam = 1.6t/m * 6m = 9.6 t , secondary beams from one side , @2.5m spacing ( 6m is instead of    2* span /2 = 2 * 6 / 2 = 6m    )

- Live load from secondary beam = 1.25t/m * 6m = 7.5 t , secondary beams from one side , @2.5m spacing

- Total loads = 9.6 + 7.5 = 17 tons @ middle of span

Mezanin management floor main beams (using metal deck allowable load ) :

- Dead load = 1.55t/m * 6m = 9.3 t, secondary beams from one side , @2.5m spacing

- Live load = 1t/m * 6m = 6 t , secondary beams from one side , @2.5m spacing

- Total load = 16 tons

Mezanin Storage Floor Secondary Beams ( out of plan beams ) :

- Dead load = 0.6t/m2 * 2.5m = 1.5 t/m

- Total Live load = 0.5t/m2 * 2.5m = 1.25 t/m

- ow = 100 kg / m

- Total Dead load = 0.1t/m + 1.5t/m = 1.6 t/m

- Total reaction = 8.5 tons

- Total Load = 2.85 t/m

- Moment = 12.8 t.m

- Required inertia = 12495 cm4

- Web depth = 380 mm

- Web thickness = 10 mm

- Flanges thickness = 10 mm

- Flanges width = 100mm

Mezanin Management Floor Secondary Beams (out of plan) :

- Dead load = 0.58t/m2 * 2.5m = 1.45 t/m

- Ow = 0.1 t/m

- Total Dead load = 1.55 t/m

- Live load = 0.4t/m2 *  2.5m = 1 t/m

- Total Load = 2.55 t/m

- Total reaction = 8 tons

- Moment = 11.5 t.m

- Web dim. = 340 x 10 mm

- Flange thickness = 10 mm

- Required inertia = 10130 cm4

- Flange length = 120 mm

Frame girder load :

- Purlins concentrated Dead loads from each sides= 2 * 0.125t = 0.25t  ,9purlins@2.5m

- Purlins concentrated Live loads from each sides = 2 * 0.4t = 0.8t ,9purlins@2.5m

- Utilities Dead loads = 0.1 t /m

Truss loads

- Purlins concentrated Dead loads at nodes from each sides = 2* 0.092t = 0.185t ,purlins@2.5m at each node

- Purlins concentrated Live loads at nodes from each sides = 2* 0.34t = 0.68t ,purlins@2.5m at each node

- Utilities Dead Load = 100 kg / node = 0.1t /node

Truss Tension Splice :

Connected box HSS 70X5

Tension force = 24 tons

Use 4 bearing bolts M10/10.9

Force on one bolt = 6 tons

Allowable force on bolt = 3.14 x 0.25 x 10.9 = 8.5 tons  ……… safe

Use plate 130 X 120 X 20

Moment on plate = 12 tons  X 1.5cm = 18 ton.cm

Plate inertia = 8 cm4

Stress on plate = 2.25 ton/cm2       < 2.4 ton/cm2     …………. Safe

Truss Compression Splice and diagonal members  :

The connected box HSS 90X5

Compression force = 24 tons

Use plate 150 x 120 x 20

Stress on plate = 24 / 144 = 0.17 ton/cm2

Moment on plate = 0.24 x ( ( 15 – 9 ) / 2 ) ) 2 = 2.16 ton.cm/cm\

Stress on plate = 2.16 X 0.5 X 1.5 = 1.62 ton/cm2         < 2.4 ton/cm2 ……….safe

Use minimum bearing bolts 2 M10/10.9

End Gable Purlins

Wind load = 46kg/m2  -> span = 2m

Wind load = 40 kg/m2  -> span = 2.5m

Use corrugated sheets of thickness = 0.5 mm , weight = 4.8 kg/m2

Used purlins :

Stairs

Live Load = 500 kg/m2

Dead load on Carriage = 100 kg/m\

Steps Calculations :

Stair width = 2.5 m

stair Tread = 267 mm

stair riser = 167 mm

distributed load on Tread = 0.267m * 0.5t/m2 = 0.134 t/m\

Moment on Tread = 0.134t/m\ * 2.52 / 8 = 0.105 t m

Shear on Tread = 0.17 tons

Use chakkar plates of steel grade 37

Required stiffness = 7.5 cm4

Chakkar plate stiffness of thickness 1 mm = 163 x 0.1 /12 = 34 cm4  -> safe

Using two Bolts to fixing Tread with Angle of Diameters = 10 mm

Stress due to moment = Mx * tp / ( 2 Ix ) = 6 Mx / ( tp2 * width ) = 0.105 * 100 / ( 1.22 * (26.7 - 2)) =  1.77 t/cm2  < ( 0.58 * fy = 2.1 t/cm2 )       ……… ok

Stress due to shear = Q / ( tp * width ) = 0.17 / ( 1.2 * ( 26.7 - 2 ) ) = almost zero

Step Fixation Angle Calculations :

Bolts construction conditions = 3 * bolts diameter * number of bolts = 60 mm

Welding length = Q / ( allowable weld stress * Sw ) = 0.17 / (0.2 * 5.2 * 0.4 ) = 0.35 cm = 3.5 mm

Use angle 50*5*5 of length 10 cm

Base connection design

use base plate of 220 x 200 x 10

base load = 2.2 tons

stress on concrete = 2.2 / (22 x 20) = 0.005 ton / cm2    -> safe on concrete bearing

applied moment on plate = 0.005 x 6.252 / 2 = 0.1 ton.cm/cm\

applied stress on plate = 0.1 x 0.5 / ( 1 / 12) =  0.6 tons/cm2   < 3.6 ton/cm2  -> safe

base welding :

use minimum weld thickness for base plate = 4 mm

plate length required = 2.2 / ( 0.2 x 5.2 x 0.4 ) + 2 x 0.4 = 6 cm

use minimum weld in both sides of web , 5cm length with 4 mm thickness

Laced Column Calculations

centroid:

Y = 76.32cm

X = 16cm

inertia:

Ix = 998867cm4 = 9.99x10E9 mm4 == 10E10mm4

Iy = 26223 cm4 = 2.62x10E8

Area = 372 cm2

ix = 51.8 cm

iy = 8.395cm

Lx (in plane length) = 10m *  2 = 20m = 2000cm ( fixed free )

Ly ( out of plane ) = 870cm ( fixed hinged )

lambda @y = Ly / iy = 103.57cm

lambda @x = 38.6cm

Fcall = 7500 / lambda-squared = 0.7 t/cm2

applied load = 133 + 91 = 224 tons

Fapplied = 0.6 t/cm2 ( safe )

End Gable

Base Design ( calculation for max column loads @ axis A +8m rigth):

Max shear on end gable base = 1.2 tons

Max Normal force = purlins own weigth x columns span x Purlins for column + column o.w + corrugated sheet ow x area

Max Normal force = 5.6 kg/m x ( 9 + 4 ) / 2  x 5 + 0.1t/m x 14m + 5.75 kg/m2 x ( 9 + 4 ) /2 * 10m Max Normal force = 2 tons

use column base steel plate = 15 x 32 cm x 1 cm

Concrete allowable stress = 75 kg/cm2

Applied normal stress = 2 / ( 15 x 32 ) = 0.0042 ton / cm2

Allowable normal force = 15 x 32 x 75 / 1000 = 34 tons       ………. Safe

Use Minimum tie rod Diameter , use  M20 / 10.9

Rshear = 0.2 x 10.9 x 3.14 x 22 / 4 = 6.8 tons

Rbearing = Diameter x plate thickness x Fu x 0.6 = 2 x 1 x 5.2 x 0.6 = 6.24 tons

Rmin = 6.24 tons ……….. safe

Use minimum weld thickness = 4mm

Use weld in web only -> weld length = ( 20 – 0.8 ) x 2 = 38 cm

Normal stress = 0.0521 ton/cm2       < 1.04 t/cm2 ………..safe

Shear stress = 0.031 t/cm2                 < 1.04 t/cm2  ………safe

Combined stresses < 1.1 x 1.04 t/cm2  ………… safe

Moment applied on plate = ( ( plate length – col height ) / 2 ) 2 x stress on concrete = 0.07 t.m/m\

Stress on plate = moment x y / Ix = 0.07 x 0.5 / (1/12) = 0.4 ton/cm2  < 2.4 ton/cm2  -> safe

Gusset plates stiffeners calculations

Buckling limit is  L / ix < 16

All gusset plates are with thickness 10 mm

Without stiffeners Lallowable = 16 x ix = 16 / ( b x t / ( b x t3 / 12 ) )0.5 = 4.5 x t mm = 45 mm

With stiffeners  , Lallowable = n x 4.5 x h mm , where n is stiffeners count , h is stiffener height and use stiffener thickness similar to plate thickness 10 mm , neglect plate actions.

use stiffener directly with minimum weld thickness 4mm and minimum weld length 5 cm in both sides